YAMAHA

# YM2163

N0 . 8 4 -  04    2

# Y M 2 1 6 3

# Digital Sound Generator (DSG)

■Summary

TheDSG(DigitalsoundGenerator)usesacpuwithbuilt-in quartz crystals to

set the initial pitch and timbre.The NMOS-LSI chip effortlessly produces

two-beat and rhythmic tones through waveform memory technology.

■Features

● 5V Single-Channel Power N-Channel E/D MOS

●TTL8-bitboard

8-pinor16-pincpusocketforcasyconnection

●5 different musical notes and 4 different rhythm sounds, can

easily produce 4 octaves scale sounds.

●Built-in7-bitD/Aconverter

●Built-in14-bittimer

●24-pin DIL socket

Japan Musical Instrument

Contents No.LSl-1121630

Manufacturing Co., LTD

YAMAHA

■Terminal

configuration diagram

# YM2163

■Block Diagram

YM2163                                                                            YAMAHA

■Terminal Function       [1]Vss Ground Terminal

Description                 [2]OR1～[5 ]OR4

The volume output terminal (orchestral sound) features a 4-channel Rex-style structure with

analog interface output. This terminal allows independent selection to output signals to either the 4-0 or 4-1 interface.

The internal source circuit design enables mixing with the rhythm sound (output from RH1 and	RH2 terminals) through an external load resistor (standard 1KΩ).

[6] IC

Clear the terminal. When the terminal is in low power mode, it resets the contents ofthe DSG internal register.

[7] NC

The two terminals inside the DSG are not electrically connected.

[8] D0～[15] D7

Data input terminal. This terminal is used to write data to the DSG internal register array and serves as an output terminal to read the status of the internal timer flag. (See Table 1)

[16] Test

This is a terminal for testing LSI. It contains a full impedance resistor. Do not connect it to anything under normal circumstances.

[17]CS

Select the chip terminal. When this terminal is set to high level, the D0 to D7 data bus terminals enter high

interference state, and the data intheDSG internal register array will be overwritten. In this case, the fault	flag status cannot be read. (See Table 1)

[18]WR

Write to the signal input terminal. Switch from CS·WR·RD= "1" to CS·WR= "0"

When a transfer occurs, the path data is written into the register array. (See Table 1)

[19] RD

Guide signal input terminal . Set CS·WR·RD to ' 1 ' to transmit data to the data bus terminal (flag status) . This status is output internally by DSG (see Table  1) .

|  | CS | WR | RD | D O～D 7 status | Notes |
| --- | --- | --- | --- | --- | --- |
| a | 1 | * | * | high saturation | *： don ’t care |
| b | 0 | 1 | 1 | Heinrichs | When a c→b or c→a transfer occurs, the path data is written into the register array. |
| C | 0 | 0 | 1 | Input (Write) |  |
| d | 0 | 1 | 0 | Output (Read) | Reading the flag status. |
| e | 0 | 0 | 0 | （ prohibit ） | Do not use this status. |

[20]IRQ

Table 1   Data Path Control

When the DSG's built-in timer's periodic delay state reverses, the CPU generates an allocation request signal (with '0' activated) and outputs it to the specified terminal.

When the flag register signal (FGR) on the data bus is set to ' 1', the flag is pulled low, and the IRQ signal returns to ' 1'. Additionally, the interrupt enable signal (IEN) is set to 0.

The status of the register array is affected by the flag parameter when writing 'O' from the data channel. The IRQ=1 flag will remain set. (The flag is masked.)

[21] RH1 [22] RH2

YM2163                                                                            YAMAHA

The RH1-type drum sound system features two primary tones, each analogically output

through the OR1-OR4 terminals. These terminals are equipped with identical S1-S2 interfaces. The load resistor is an external component with a nominal value of 1kΩ .

[23]  VDD

Power terminal (nominal voltage +5V)

[24]  ∮

Clock pulse (standard 1 MHz) input terminal.

. function declaration

All DSG functions are managed by a 23-register array, with the CPU controlling these functions through the register array.

Simply import the data into the register instruction, and the DSG system will automatically

generate the sound. According to the configuration of each sound block, the sound will be played in sequence.

●Interval divider: a precision device for adjusting musical intervals.

●Waveform ROM: Stores musical waveforms and supports lossless audio.

● Envelope Generator: Can generate envelope sound effects.

● Multiple tracks: Create sounds with additional envelopes.

● Rhythm oscillator: generates the frequency components of rhythmic sounds.

● Rhythm Envelope Generator: Add low-frequency effects to rhythm sound effects.

●D/A converter: Converts computational results into digital signals, while also digitizing musical and rhythmic sounds.

In addition, the timer generates a CPU 8x frequency division request signal by calculating the period time set in the register array.

As shown in Table 2, the register array stores the following data: When the most significant bit (D7) of the data channel DO ～D7 is set to  1, the address is written into the DSG's internal register array. If D7 is set to 0, the previously written addresses D0 ～ D6 are cleared. Written data is not overwritten by default (as indicated by the asterisk (*) in Table 2, even when set to'

1 ', it automatically resets to' 0' after a specified period).

YM2163                                                                            YAMAHA

;

*Write data

| address | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | COMMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80H 81H 82H 83H | 0 0 0 0 | DV4 (0) (1) (2) (3) | DV3(0) (1) (2) (3) | DV2(0) (1) (2) (3) | DV1(0) (1) (2) (3) | DV0(0) (1) (2) (3) | DV1/2(0) (1) (2) (3) | DV1/4(0) (1) (2) (3) |  | Weeks (lower) |  |  |  |  |  | Music (Wind and String) |
| 84H 85H 86H 87H | 0 0 0 0 | KON(O ) (1) (2) (3) | FD(0) (1) (2) (3) | B2 (0) (1) (2) (3) | B1(0) (1) (2) (3) | DV7(0) (1) (2) (3) | DV6(0) (1) (2) (3) | DV5(0) (1) (2) (3) |  | Button | Full octaves | octave |  | Weeks (upper) |  |  |
| 88H 89H 8AH 8BH | 0 0 0 0 | E2(0) (1) (2) (3) | E1(0) (1) (2) (3) | SUS(0) (1) (2) (3) |  | W3 (0) (1) (2) (3) | W2 (0) (1) (2) (3) | W1 (0) (1) (2) (3) |  | Envelope Selection |  | Keep on |  | Waveform memory |  |  |
| 8CH 8DH 8EH 8FH | 0 0 0 0 | Test 1 2 3 4 | VL2(0) (1) (2) (3) | VL1(0) (1) (2) (3) | F4 (0) (1) (2) (3) | F3 (0) (1) (2) (3) | F2 (0) (1) (2) (3) | F1 (0) (1) (2) (3) |  | test | Volume selection |  | Output filter selection |  |  |  |
| 90H 91H 92H 1/4 93H | 0 | FGR | IEN | HHD · | HHO · | SDN | HC · | BD · |  |  |  | Rhythm Trigger |  |  |  | Section Sound |
|  |  |  |  |  |  |  |  |  |  | Effective squeeze Logo Settings |  |  |  |  | Maintain rank |  |
| 94H 95H 96H 97H | 0 0 0 0 |  | LV4(0) (1) (2) (3) | LV3(0) (1) (2) (3) | LV2(0) (1) (2) (3) | LV1(0) (1) (2) (3) | LV0(0) (1) (2) (3) | LH (0) (1) (2) (3) |  |  | (HH) (BD) (HC) (SDN) |  | Rhythm Level |  |  |  |
| 98H 99H 9AH 1/4 9BH | 0 | PT6 | PT5 | PT4 | pT3 | PT2 | PT1 | PT0 |  | Timer period (lower bit) |  |  |  |  |  | Timer |
| 9 CH 9 DH 9 EH 1/4 9 FH | 0 | PT13 | PT12 | PT11 | PT10 | PT9 | PT8 | PT7 |  | Timer period (upper) |  |  |  |  |  |  |

Note: Add characters (0) to (3) indicate the pronunciation channel.

* read data

| address | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | COMMENT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| don ’t care |  |  |  |  |  |  |  | FLAG | state |

Table 2   DSG Address Mask

YAMAHA                                                                 YM2163

(1) Frequency and Base Ion  · Disable data settings (addresses 80H ～ 87H)

When the number of weeks is set to DV7～DV1/4 and the octave values are B1, B2, the fundamental frequency f

| B2 | B | short-cut process |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 2 |
| 1 | 0 | 3 |
| 1 | 1 | 4 |

Th i s  wi l l  be  det erm in e d  her e .

| Weeks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| superior |  |  | inferior |  |  |  |  |  |  |
| DV7 | DV6 | DV5 | DV 4 | DV 3 | DV 2 | DV1 | DV0 | DV1/2 | DV1/4 |
| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 1/2 | 1/4 |

∮: DSG main clock frequency

In  K ON ' s  dat a  configuration ,  rhythm  is  controlled  by  the  onset  an d  offset  of

syll able  pronunciation .  FD ,  however ,  employs  a  decay  mechanism

| KON |  |
| --- | --- |
| 0 | Key Off (Start attenuation) |
| 1 | Press to start (sound starts) |

independent  of K ON .

| FD |  |
| --- | --- |
| 0 |  |
| 1 | Shape-Shifting Damper (Forced Damping) |

(2) Configuration of waveform memory envelope (addresses 88H to 8BH)

Sound  waves  and  envelopes  will  be  selected  ba sed  on  the  following  dat a .

|  | wavememorizer |  |  |
| --- | --- | --- | --- |
| W 3 | W 2 | W 1 |  |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 | St（string music ） |
| 0 | 1 | 0 | or (organ) |
| 0 | 1 | 1 | Cl（clarinet ） |
| 1 | 0 | 0 | Pf (Xianuo) |
| 1 | 0 | 1 | Hc (half fourth) |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |

( Re fer  to  Figur e  1 )

|  | Envelope Selection |  |
| --- | --- | --- |
| E2 | E1 |  |
| 0 | 0 | Envelope 0 (attenuated sound) |
| 0 | 1 | Envelope 1 (sustained tone) |
| 1 | 0 | Envelope 2 (sustained tone) |
| 1 | 1 | Envelope 3 (sustained tone) |

(Refer to Figure 2)

| Continueto specify |  |
| --- | --- |
| SUS |  |
| 0 | Keep Off |
| 1 | Keep on |

(Refer toFigure 2)

YM2163                                                           YAMAHA

Hc                                  Pf                                  CL                               Or                                 St

Figure 1  Waveform Memory (Melody)

|  | SUS=0(keep off) | SUS=1(keep on) |
| --- | --- | --- |
| E n v e l o p e 0 |  |  |
| E n v e l o p e 1 |  |  |
| E n v e l o p e 2 |  |  |
| E n v e l o p e 3 |  |  |

Figure2  Envelope (Melody)

YAMAHA                                                               YM2163

## (3) Volume and Filter Selection Settings (Addresses 8 CH to 8 FH)

| Volume (water chime) |  |  |
| --- | --- | --- |
| V2 | V1 |  |
| 0 | 0 | 0 dB (normal) |
| 0 | 1 | -6dB |
| 1 | 0 | -12dB |
| 1 | 1 | -∞dB |

Volume adjustment data and output terminal designation data will be set as follows.

| Output terminal selection (filter) |  |  |
| --- | --- | --- |
| F1 | 0 | Do not output to OR1 terminal. |
|  | 1 | Output to OR1 terminal. |
| F2 | 0 | Do not output to OR2 terminal. |
|  | 1 | Send the output to the OR2 |
| F3 | 0 | Do not output to the OR3 terminal. |
|  | 1 | Output to OR 3 terminal. |
| F4 | 0 | Do not output to OR4 terminal. |
|  | 1 | Output to OR4 terminal. |

## (4) Rhythm Trigger (Addresses 90H to 93H)

When the data " 1" is deviated, the system will emit the following rhythm sound effect, and the

built-in rhythm envelope generator will naturally change, and the data will automatically recover to "0".

| Rhythm Tracker |  |  | explain |
| --- | --- | --- | --- |
| BD | 0 |  | Output the signal to terminal RH1. |
|  | 1 | Ding |  |
| HC | 0 |  | Output the signal to the RH1 terminal. |
|  | 1 | Speak at high volume |  |
| SD N | 0 |  | Output the signal to the RH2 terminal. |
|  | 1 | Military drum noise |  |
| HHO | 0 |  | Output the signal to the RH2 terminal. |
|  | 1 | High Hat (Open) Pronunciation |  |
| HHD | 0 |  | Output the signal to the RH2 terminal. |
|  | 1 | Hat (end) pronunciation |  |

## (5) Rhythm hierarchy (address 94H～97H)

When controlling the rhythm output rate through the DSG external device, the rate conflicts with the

| Rhythm Level |  |  |  |  |
| --- | --- | --- | --- | --- |
| LV4 | LV3 | LV2 | LV1 | LV0 |
| 16 | 8 | 4 | 2 | 1 |

built-in rhythm envelope generator. In this case, the

| Level hold |  |
| --- | --- |
| LH |  |
| 0 | Do not keep the set rhythm level |
| 1 | Maintain the set rhythm level |

e is typically set to 1.

The volume level ranges from 0 (maximum) to 31 (minimum).

(6) Timer interrupt control (addresses 98H-9F H, 90H-93H)

The DSG's internal interrupt flag is set every time the preset timing period is reached.

时间周期PT  Here, ∮: DSG's main clock frequency (Hz)

| superior： |  |  |  |  |  |  | F Position |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pT13 | P T12 | PT11 | PT10 | PT9 | PT8 | PT7 | PT6 | PT5 | PT4 | PT3 | PT2 | PT1 | PT0 |
| 8192 | 4096 | 2048 | 1024 | 512 | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |

The interrupt flag is reset when the reset signal FGR= "1" is written, and FGR automatically returns to "0" at this time.

When the distribution completion flag is in an interrupted state, the IEN signal will turn on. In this case, the IRQ terminal will output an inverted signal, and the D0 terminal will also output a corresponding

signal in read mode.

YM2163                                                                YAMAHA

. electricalcharacter

Absolute maximum freeze

| mark |  | min | sta | max | unit |
| --- | --- | --- | --- | --- | --- |
| VI | terminal voltage | -0.3 |  | 7 | V |
| Top | operating temperature | 0 |  | 70 | ℃ |
| Tstg | Save temperature | -50 |  | 125 | ℃ |

DC Characteristics (@VDD=4.5~5.5V)

| mark |  | min | sta | max | unit |
| --- | --- | --- | --- | --- | --- |
| VDD | supply voltage | 4.5 | 5.0 | 5.5 | V |
| IDD | Power supply current @RL= 1KΩ |  | 25 | 45 | mA |
| VIL | Enter low voltage ( ∮、C S 、D0-D7、W R 、R D 、 I C） |  |  | 0.8 | V |
| VIH | Enter high-level voltage ( ∮、C S 、D0-D7、W R 、R D 、 I C） | 2.0 |  |  | V |
| VOL | Output low voltage (D0, IRQ) @IOL= 1.6mA |  |  | 0.4 | V |
| VOH | Output high-level voltage (D0) | 2.4 |  |  | V |
| ILK | Enter leakage current ( ∮、 C S 、D 0-D 7 、W R 、R D 、 I C）@VI=5V |  |  | 10 | μA |

AC characteristic

| mark |  | min | sta | max | unit |
| --- | --- | --- | --- | --- | --- |
| Tcw | Chip selection in pulse mode (Figure 3) | 135 |  |  | ns |
| Tww | In the optical signal pulse (Figure 3) | 135 |  |  | ns |
| TDS | Data Set Time (Figure 3) | 160 |  |  | ns |
| TDHW | Data retention time (write) (Figure 3) | 30 |  |  | ns |
| TACC | Access Time (Figure 4) |  |  | 400 | ns |
| TDHR | Data retention time (read) (Figure 4) | 0 |  |  | ns |
| f∮ | Clock frequency (Figure 5) | 0.9 | 1.0 | 1.1 | MHz |
| T∮ON | Clock enable time (@f∮= 1MHz) (Figure 5) | 400 |  |  | ns |
| T∮OFF | Clock shutdown time (@f∮= 1MHz) (Figure 5) | 400 |  |  | ns |
| T∮R | Clock up time (Figure 5) |  |  | 80 | ns |
| T∮F | Clock down time (Figure 5) |  |  | 80 | ns |
| TOD | ——— Output delay time IRQ (Figure 5) |  |  | 700 | ns |
| Vp-p | Maximum Output Vibration (OR1-4, RH1-2) @RL= 1KΩ, VDD=5V (Figure 6) | 0.60 | 0.90 | 1.40 | V |

YAMAHA

Figure 3: Write time

—— C S

YM2163

Tcw

——

WR

Tww

D0～D7

TDS    TDHW

— —

——

Note: The TDS and TDHW criteria are determined when either CS or WR reaches a high level.

Figure 4: Read time

——

CS

——

RD

D0～D7 ·

TAcc                    TDHR

——    ——

Note: TACC C S uses DR the party with lower rank and slower response as the reference.

— —          — —

TDHW is CSbased DRon the time when one of the parties reaches the higher level.

YM2163                                                                           YAMAHA

Figure 5 Clock/Output Timing

f∮

T∮DN             T∮R       T∮OFF           T∮F

I RQ

TOD

Figure 6 Maximum output amplitude

OR1

OR4

(Note) When W1= 1 , W2= W3= 0 , VL1= 0 , and VL2= 0 , this rule applies .

YAMAHA                                     YM2163

■ Dimension

Note:product specifications are subject to change without notice due to improvements or other reasons

Agency                          — 一 JapanMusicalInstrumentManufacturingco,Ltd.—

Kanematsu semiconductor co.,Ltd.

1-6-1 Shin'fu, Chuo-ku, Tokyo, Japan

Tel:(03) 551-7791

■Our company

contact

■Fukuyama

P.C.430

10-1Nakazawa-cho,Hama matsu shizuoka prefecture TEL: 0534-65-1111

P.C. 438  01

203 Matsunokijima, Toyooka Village, Iwata District

Shizuoka Prefecture

TEL 053962-3111